SIMULATION OF ELECTRON BEAM BUNCHING PROCESS IN RELATIVISTIC MICROWAVE AMPLIFIERS FOR TWO BEAM ACCELERATOR

L.V.Bobyleva, A.V. Elzhov, V.I. Kazacha, E.A. Perelstein,

Joint Institute for Nuclear Research, Dubna, Russia

INTRODUCTION

Some schemes for generating of high-intensity drive beam bunches in the two beam accelerator (TBA) concept are being studied now [1-6]. This work is devoted to the computer simulation and comparison of the bunching of a relativistic electron beam produced by a FEL and by a travelling wave tube (TWT). The calculation results of the electron beam bunching in the FEL (with and without guide magnetic field) also are compared with the experimental results in [4, 6].

BEAM BUNCHING EQUATIONS

The following equations describe the self-consistent spatial problem of the movement of a relativistic electron beam in a microwave electromagnetic field, which can be used both for a TWT and  for a FEL in the Compton regime:

dgjdZ=-kassinyj   (j = 1, 2, …, M),

dqjdZ=1β||j−1βph, dasdZ=h<sinyj>,

djdZas=h<cosy>.                     (1)

Here M is the number of electrons (macroparticles), gj is the jth electron relativistic factor; m0 is the electron rest mass, Z = zw0/c is the dimensionless longitudinal coordinate, and w0 is the microwave frequency. The value qj  is the phase of the jth  electron relative to the electromagnetic field; j is the phase of the microwave complex amplitude (a=aseij), yj = j + qj  is the total ponderomotive phase. The brackets in the  equations (1) denote an average over the bucket.  The value as=eEs/m0w0c is the dimensionless amplitude of the microwave electric field, Es is the amplitude of the microwave electric field. The parameter k is the microwave coupling coefficient. Its value depends on the type of used device. The parameter h=IbIA⋅2kN, the constant IA =  m0c3/e @17 kA;  N is the norm of the electromagnetic wave [7]; β||j is longitudinal electron velocity and βph is the microwave phase velocity.

SIMULATION OF BUNCHING PROCESS IN FREE ELECTRON LASER

To obtain the system of differential equations for simulation of the bunching process in a FEL with a helical wiggler  we have used the corresponding equations [7,8], taking into account the effective frequency shift wp connected with the plasma wave in a beam. It is similar to the accounting of a plasma wave in the resonance condition in [4]. Finally we get:

dwjdZ=k1jassinyj                          (j = 1, 2, …, M ),

dqjdZ=-(kw+ks)⋅cw0+1+wpw0βz+1+aw2+awascosyj2g02(1−wj)2, dasdZ=h1<sinyj1−wj>,djdZas=h1<cosyj1−wj>, (2)

where k1j=aw2g0gj=k01−wj.Here wj=1−Ej/E0 is the relative change of the jth electron energy; kw=2p/λw;λw is the wiggler period; ks  is the axial wavenumber for the waveguide resonance mode inside the wiggler; g0 is the electron initial energy; βz is the longitudinal dimensionless velocity of the resonant particle, determined from the following relations: βz=β02−β2, β/βz=aw/g0; k0=aw/2g02 and wp=4pnee2m0⋅1gzg12 is the relativistic plasma frequency, ne is the electron beam density and gz=11−βz2. The parameter aw=eBwλw/2pm0c2 , where Bw is the wiggler magnetic field amplitude. The coefficient h1=IbIA⋅2k0g0N.

For our simulation we used the electron beam and FEL parameters from [4]: electron beam energy                     ~2.2 MeV, electron current inside the wiggler  ~500 A, electron beam radius ~0.5 cm, wiggler period - 12 cm, wiggler field Bw = 1.1 kG,  microwave frequencyf0 = 3.5×1010 Hz (H11 mode). In [4] the initial microwave power   of   10  kW   was  injected   into  the  circular waveguide in the H11 mode. The corresponding microwave electric field amplitude  as@4.4⋅10−4. Then we obtain the following parameter values: k1@0.022,h1=2.5⋅10−4  (the norm of wave N = N11@ 24.3 in our case). We define the bunching parameter as: B  = |<eiy>|.

We also simulated the initial energy spread in the electron beam with gaussian distribution and the relative energy dispersion s.

Fig.1. The FEL microwave power versus the length.

Fig.1 shows the results of the calculations of the FEL microwave power and their comparison with the results obtained in [4]. To take into account the wiggler adiabatic entrance in [4], we suppose the exponential power growth to start at 48 cm from the wiggler beginning. This point corresponds to zero length in Fig.1. As one can see from Fig.1, these results are in a rather good agreement. The difference observed between the maximum microwave power in [4] and the calculated one for monoenergetic beam in ~10 times may be partially due to the initial energy spread in the beam.

Fig.2 shows the calculated spatial evolution of the bunching parameter B. The vertical lines in Fig.2 correspond to the time dependence measurements of the bunching parameter along the beam [4]. The regular wiggler part ends at the length approximately equal to 170 cm. Our further along the wiggler calculations are not sufficiently correct. So comparing the obtained curve corresponding s  = 5% and the experimental results one can suppose that the low experimental bunching parameters in [4] (B ~0.1) were due to the beam energy spread influence. The maximum bunching is reached in the region of maximum micromave power values.

Fig.2. The FEL bunching parameter versus the length.

To simulate the beam dynamics in a FEL with guide magnetic field we used  the system similar to (2). The only difference was in the second equation, which with account of [9] looked as

dqjdZ=-(kw+ks)⋅cw0+1+wpw0βz+1+aw2+awascosyj2g02(1−wj)2,.    (3)

Here the parameter βz0=β0/1+λ2, F0 value is given by the formula [9]

F0=,

where λ=±vvz; Ω0=; B0 is the guide magnetic field.

The λ value was obtained from the following equation [10]

β0λ21+λ2=eB0m0g0ckw⋅λ2+2eBwm0g0ckw⋅(1+,

where I1(λ) is the modified Bessel function.

For our simulation we used the following electron beam and FEL parameters from [6]: electron beam energy  ~1.5 MeV (g0=4,β0=0.968); electron beam radius - 0.3 through 0.5 cm; electron current inside the wiggler - 50 through 70 A; wiggler period λw = 7.2 cm; wiggler field Bw = 2.1 kG; reversal axial magnetic field Bw = -1.4 kG, microwave frequency B0 =f0 Hz (N01 mode); initial microwave power at the wiggler input - 6 through 20 kW; microwave power at the wiggler output ~5 MW; waveguide radius                                            ~1.5 cm; wiggler length ~250 cm; length of the microwave saturation  ~120 cm; initial energy spread in the  electron  beam  -  1%  through  2%. For the chosen value of reversal magnetic field B0 = -1.4 kG. we obtain βz0 ~ 0.912, βz0 ~1.41; 4.4⋅10−2; β0@0.968; k@4.4⋅10−2, and k@4.4⋅10−2.

The final results of our calculations are shown in Fig.3. The curves 1,2,3 and 4 were calculated for different initial energy spread values in the electron beam s = 0, s = 3%, s = 5% and s =7%, respectively.

Fig.3. The FEL microwave power versus the length at axial magnetic field B0 = -1.4 kG.

The experimental results [6] are shown by the dots. They rather good fit the theoretical results at s =7%.

SIMULATION OF BUNCHING PROCESS IN TRAVELLING WAVE TUBE

To simulate the electron beam bunching in a TWT amplifier based on the corrugated waveguide, we used the following system of differential equations:

dwjdZ=k2assinyj      (j = 1, 2, …, M),        dqjdZ=1−wj(1−wj)2−g−2−1βph,          (4)

dasdZ=h2<sinyj>, djdZas=h2<cosyj>, where k2=pl0g0d,  l0  is the corrugation amplitude and  d is the corrugation spatial period [11].  The parameter h2 is the following: h2=IbIA⋅2k2g0N01, where N01 is the norm of E01 type wave in the TWT.

Fig 4. The TWT micrrowave power versus the length

We chose the following electron beam and E01 type electromagnetic wave parameters for simulation: electron beam energy ~2.2 MeV, electron current inside the TWT ~500 A, electron beam radius ~0.5cm, microwave frequencyf0 = 17×109 Hz (λ~1.76 cm), initial microwave power in TWT=10 kW. The parameter d value was found from the dispersion curve of the corrugated  waveguide  having l0 = 1mm, radius ~1.8 cm and  at βph ~ 0.982. The dispersion curve was calculated with the help of the code URMEL. In our case  d  was ~5.8 mm; ksd=2p3. Then we have  k2»0.102 and h2»3.2⋅10−3 (the norm of the wave N01~ 10 ). For the initial microwave power ~10 kW in the TWT the corresponding dimensionless amplitude of the  microwave  electric  field  as ~ 6.8 ×10-4.

Fig.4. represents the calculated evolution of the microwave power as a function of the interaction length along the TWT. The two curves correspond to the initial electron beam energy 2.2 MeV and 1 MeV.

One can also see from Fig.4 that the bunching length in the TWT decreases essentially when the initial beam energy is diminished. The dependence of the microwave power at the TWT exit on the energy spread (2.2 MeV) is rather weak. This dependence becomes sufficient when the initial electron beam energy decreases down to 1 MeV. As one can see from Fig.1 and Fig.4, the bunching length of the electron beam with energy 2.2 MeV is in  ~1.5 times less in the TWT than in the FEL. The energy spread in the bunches is larger in the TWT than in the FEL.

CONCLUSIONS

Our simulations showed that a high degree of bunching of the electron beam having the energy in the range 1 - 2 MeV can be rather easily achieved in a short travelling wave tube. 	The electron beam (2.2 MeV, 500 A) bunching occurs in a TWT at the distance in  ~1.5 times shorter than in a FEL. It is connected with the fact that the space gain in the TWT is much higher than that in the FEL. But the bunch energy spread is larger in the TWT than in the FEL. The bunching effect depends weakly on the initial energy spread in the TWT and greatly in the FEL.

The TWT bunching efficiency decreases with the initial electron beam energy increase. Thus the TWT may be efficiently used for the electron beam bunching in the energy span 1 MeV through 2 MeV when the TWT length may be made of 0.5 m through 1 m.

The simulation and experimental results for FEL with guide magnetic field as well as without it are in rather good agreement that confirms our model application for description of the electron beam bunching in power relativistic amplifiers.

REFERENCES

[1] A.M. Sessler, Proc. Workshop on the Laser Acceleration of Particles, eds. C. Yoshi and T. Katsouleas, AIP Conf. Proc. 91 (1982), p. 154.

[2] W. Schnell, CERN-LEP-RF/86-27 (1986); CERN-LEP-RF/88-59 (1988).

[3] G.Fiorentini, T. Houk and C. Wang, UCRL-JC-111391, Livermore (USA, 1993).

[4]  J. Gardelle et al., Preprint CESTA/5 (1996).

[5] J. Gardelle, J. Labrouche and J.L. Rullier, in: the Proc. of EPAC 96 (Sitges (Barcelona), 10-14 June 1996), v.1, p.298.

[6] Yu.B. Viktorov et all., Proceedings of the XI All Union Meeting on Accelerators of Charged Particles, v.2 (1989), p.95.

[7] N.S. Ginzburg and A.S. Sergeev, Zh. Tekhn. Fiz., v.61 (1991), p. 133.

[8] A.M. Sessler et al., Nucl. Instr. Methods, A 306 (1991), p. 592.

[9] C.W. Roberson and Sprangle, Phys. Fluids B, v.1, No.1 (1989), p.3.

[10] H.P. Freund, S. Johnson, P. Sprangle, IEEE Quan. Elect., QE-19 (1983), p.32.

[11] V.L.Bratman et al., in: Relativistic High- Frequency Electronics, ed. A.V. Gaponov-Grekhov (IAP AS USSR, Gorky, 1979), p. 249.