NEW SCHEME OF TWO BEAM ACCELERATOR DRIVER ON BASE OF LINEAR INDUCTION ACCELERATOR

A.V. Elzhov, V.I. Kazacha, E.A. Perelstein

Joint Institute for Nuclear Research, Dubna, Russia

INTRODUCTION

Some schemes of the TBA driver based on a linear induction accelerator (LIA) were discussed in [1-4]. In these schemes the driver electron beam moves through an alternating discrete row of microwave generators (free electron lasers (FEL), relativistic klystrons, etc.) and reaccelerator sections. The microwave power is totally extracted from the driver after every generation section. Such kind of driver was experimentally investigated in [5], where the re-acceleration of a modulated with the chopper help electron beam through two induction accelerator cells was demonstrated. The continuous extraction  along the whole driver length of the microwave power was designed in CLIC [6] and a corresponding beam dynamics was studied also in [7].

A new scheme of two beam accelerator (TBA) driver based on a linear induction accelerator is suggested in this work. The scheme is quite uniform and has the following characteristic properties: a) the electron beam bunching occurs at a rather low ~1 MeV initial energy; b) the bunched beam further acceleration occurs in the accompaning enhanced microwave that provides the steady longitudinal beam bunching along the whole driver; c) there is no total microwave power extraction anywhere in the driver; d) a waveguide is used along the driver.

The driver consists of an injector, buncher and long (a few hundreds of meters) row of separate LIA sections producing the external accelerating electric field and partitioned by transition chambers. The injector produces the initial electron beam with energy 1 through 2 MeV and current 0.5 through 1 kA. This beam is injected into the buncher. It may be a travelling wave tube (TWT) working in the amplification mode. It was shown in [8] that a high degree of bunching can be rather easily achieved in the TWT at the distance of ~1 m. Then the electron bunches continue moving in the LIA in accompaniment of the enhanced in the TWT microwave and simultaneously are accelerated in the LIA electric field. The microwave power extraction from the driver occurs only in the transition chambers. The system attains the steady state at first few tens of meters where the bunch energy increases up to the level of ~10  MeV. Then the section of quasi-stationary microwave generation begins where the total power, that the accelerating field inserts into the beam, transforms into the microwave power.

The scheme has the following merits: 1. The possibility to provide the microwave phase and amplitude stability. The phase stability can be obtained at the expense of quasi-continuity of the system. It means that only mechanical tolerances will be of consequence; 2. Due to the bunched beam acceleration  it is not necessary to have a high (~10 MeV) energy of the initial electron beam bunching.

SIMULATION OF ACCELERATION OF ELECTRON BUNCHES ACCOMPANIED BY ELECTROMAGNETIC WAVE IN EXTERNAL ELECTRIC FIELD

We have three characteristic regions of the driver: 1) the bunching region with using of TWT and without acceleration, 2) transition region with a beam acceleration and 3) quasi-stationary beam propagation region.

As it was shown in [8], one can obtain the electron beam bunching in a TWT at a rather short length ~50 cm for the following electron beam and E01 type microwave parameters:

electron beam energy             ~ 2.2 MeV   (g0  ~5.31)

electron current inside TWT                       Ib ~500 A

electron beam radius                                      ~0.5 cm

microwave frequency  f0 =17×109 Hz (λ~1.76 cm)

initial microwave power in TWT                    10 kW.

output microwave power in TWT

at the acceleration region entrance               13 MW.

Without accompanying microwave, the debunching process will immediately occur at the distance of about few tens of centimeters. The simulation showed that the electron bunches can be transported at the distance ~10 m if the bunch movement is accompanied by the microwave amplified in the TWT. The most encouraging situation is when the moving after the TWT output bunches are accompanied simultaneously by the amplified microwave and are accelerated in the external electric field. inside a corrugated waveguide. The waveguide must be transparent for a pulse accelerating field and a good screen for the microwave.

The following system of differential equations was used for the simulation of the bunched beam acceleration after the TWT output [7], [8]:

dwjdZ=k2assinyj,        (j = 1, 2, …, M)

dqjdZ=1β||j−1βph, dasdZ=h<sinyj>-,

djdZas=h<cosy>,                                      (1)

where M is the number of electrons (macroparticles) used in our simulation; wj=1−Ej/E0 is the relative change of the jth electron energy,.E0 is an initial electron energy, Z=zw0c is the longitudinal dimensionless coordinate; w0 is the microwave frequency; β||j is the longitudinal dimensionless electron velocity, and βph is the microwave phase velocity. The value qj is the phase of the jth electron relative to the electromagnetic field; j  is the phase of the microwave complex amplitude (a=aseij), yj = j + qj is the total ponderomotive phase. The brackets in the  equations denote an average over the bucket. We define the bunching parameter B as B=. The valueasis the dimensionless amplitude of themicrowave electric field as=eEs/m0w0c, Es is the amplitude of the microwave electric field. The parameter e0=eE0/m0w0cg0  is the dimensionless value of the external electric field Ea: The parameter G is the attenuation constant characterizing the distribution of the microwave losses along the driver.

The microwave coupling coefficient was taken  to  be  k»0.102;  the  parameter h»10−3 corresponds to the above mentioned beam parameters and the E01type electromagnetic wave. For our calculations we took the value of the accelerating electric field Ea = 1 MV/m that had been practically achieved in the LIA [9].

To provide the quasi-stationary beam transition along a driver we can evaluate the necessary steady state parameters from (1). Taking into account that the beam is bunched in the region of y»p we obtainy@aske0, G@-hdas and  dqjdZ=1β||j−1βph.

To estimate the values of G@ and βph we took the amplitude as value from the TWT bunching process calculation and β||j0.9989. Then we have G@10−3 and phase velocity in the steady state: β@1.03. The beam has to be accelerated from the energy  ~2 MeV up to the energy 10 MeV. It means that the microwave phase velocity βph must increase from β0@0.982 up to β@ 1.03, for example by a rule

βph(Z)=β0+(β−β0)(1−exp(−kZ)),

where the k value was chosen  to be 10−3. To provide the space changing of the phase velocity we must vary a corrugation amplitude, spatial period and a mean waveguide radius upon the condition that the coupling constant does not change according to the above mentioned considerations.

SIMULATION RESULTS

The real Gand βparameter values, when the steady state of the system actually occurs, were found during the process of several iterations. Their final values turned out to be  G@10−3 and β@ 1.02. The first set of calculations was accomplished for the case when the external electric field Ea and the attenuation constant G had been continiously distributed along the driver. The particle parameters (gj and yj) obtained at the TWT output were taken as the input data for the driver steady state calculations.

Fig.1.The electron mean energy versus the distance for: 1-continuously distributed driver parameters; 2-discrete periodic cells, 3-discrete cells with narrow accelerating gaps.

Our calculations showed that the bunching process in the TWT must be interrupted some earlier the moment when the bunching parameter B attains its maximum value. Then we have no particle losses in our further calculations in the driver. Fig.1 shows the calculated dependence of the bunch mean energy g on the distance (z = 0 corresponds to the TWT output). The dependences of the microwave power and the bunching parameter B on the distance are depicted in the Fig.2 and Fig.3. As one can see from these figures, the steady state of the system is achieved at the distance  ~40 m. The extracted microwave power in the steady state is equal to ~333 MW. The mean steady state bunch energy ~7.5 MeV. And at last we have rather high bunching parameter B@0.9 in the steady state. The electron phase picture (see Fig.4) shows that the initial bunch breaks down on two main bunches being at Dy@2p one from another.

Fig.2. The microwave power versus the distance for continuously distributed driver parameters.

The power that external electric field puts into the electron beam is equal to ~500 MW/m. And it transforms into the microwave power in the steady state.

Fig.3. The bunching parameter versus the distance for continuously distributed driver parameters.

Figs. 4 and 5. The typical electron phase space picture at the distance of 100 m for continuously distributed driver parameters and the microwave power versus the distance from 100 m through 110 m for discrete periodic cells.

The stability of the steady state solution with respect to the possible error in the electron beam current Ib was also investigated. The calculations showed that we still obtained the steady state solution up to error values dII@±10%.

A real LIA always consists of separate induction sections partitioned by transition chambers, so that it has a discrete structure. The second set of our calculations was performed for the realistic induction section parameters. As a result, the optimal lengths of the induction sections (ls = 50 cm) and transition chambers (lt = 25 cm) were found. The microwave extraction was switched on only in the transition chambers.. And the electric field Ea was equal to 1.5 MV/m inside the accelerating sections.

The dependence of the bunch mean energy on the distance is depicted in the Fig.1 (curve 2). One can see a short range of dependence of the microwave power along the distance of 10 m (from z = 100 m through  z = 110 m) in Fig. 5.

In the third set of calculations for the same lengths of the accelerating sections and the transition chambers the only difference was the following: accelerating potential of every section (750 kV) was concentrated only on the gap 7.5 cm long located at the beginning of every section to make more simple waveguide in this case. The dependence of the bunch mean energy on the distance is depicted in the Fig.1 (curve 3). The steady state occurs also when the system has the discrete structure. The bunching parameter B stays also high along the driver (B@ 0.9). The microwave power amplitude stays rather stable.

CONCLUSIONS

1. Our simulation showed that a steady state can be found when electron bunches accompanied after a TWT by an amplified microwave are simultaneously accelerated in an external electric field both for continiously distributed system parameters and for discrete parameters of accelerating sections.

2. The total power, which is inserted into the beam by the accelerating field, transforms into the microwave power in the steady state. It was equal to ~500 MW/m in our case. Such kind of systems can serve effectively as a rather long (hundreds of meters) driver for the TBA concept.

REFERENCES

[1] A.M. Sessler, Proc. Workshop on the Laser Acceleration of Particles, eds. C. Yoshi and T. Katsouleas, AIP Conf. Proc. 91 (1982), p. 154.

[2] A. M. Sessler et al., Nucl. Instr. Meth., A306, (1991),.p.592.

[3] T. Houck et al., IEEE Trans. on Plasma Science, vol.24, 3(1996), p.938.

[4] G.G. Denisov et al., Nucl. Instr. Meth., A358 (1995), p.528.

[5] T.L. Houck and G.A. Westenskow, Preprint UCRL-JC-119021, LNLL, Livermore, California, USA, 1994.

[6] W. Schnell, CERN-LEP-RF/86-27 (1986); CERN-LEP-RF/88-59 (1988).

[7] N.S.Ginzburg et al., in: Relativistic High-Frequency Electronics, v.5 (IAP AS USSR, Gorky, 1988), p. 37.

[8]  Perelstein E. A. et al, JINR Preprint E9-97-5, Dubna, 1997.

[9] O.V.Arkhipov et al., JINR Preprint E9-91-50, Dubna, 1991.

.